# Hydrology-Based Flood Discharge & HFL Estimation – EPC SaaS Reference Note

## Purpose of This Document

This document explains how empirical flood-discharge formulas and High Flood Level (HFL) determination are practically used in Indian EPC hydrology projects, and how these methods can be automated into a B2B SaaS platform.

## What the Framework Represents

The methodology is not a single model but a decision framework combining empirical formulas, basin-wise engineering practice, terrain classification, and regulatory defensibility. It is primarily used for ungauged or sparsely gauged river reaches.

## Why Multiple Flood-Discharge Formulas Exist

Flood-discharge formulas are empirical and region-specific. Each formula is valid only under certain physiographic and rainfall conditions. Using an incorrect formula may lead to technically or legally unacceptable designs.

## Core Flood-Discharge Formulas – Practical Meaning

Creager: Extreme floods in steep Himalayan and Western Ghats catchments.
Dicken: Large monsoon-driven alluvial rivers.
Fanning: Transitional rolling or plateau terrain.
Fuller: Statistical validation and extreme flood checks.
Inglis: Deccan plateau rivers.
Ryves: Short, steep Western Ghats rivers.
Observed HFL: Governing parameter wherever available.

## Critical Engineering Rule

Empirical formulas estimate flood discharge only. HFL must be determined using observed data, gauge correlation, or hydraulic conversion. Formulas support HFL estimation but do not directly define it.

## Practical EPC Workflow

1. Classify location (basin, terrain, tidal influence).
2. Determine permissible methods.
3. Estimate design discharge using approved formulas.
4. Convert discharge to HFL using hydraulic or gauge-based methods.

## HFL Determination Hierarchy

1. Observed gauge HFL.
2. Gauge correlation.
3. Hydraulic computation.
4. Empirical formula support only.

## Why EPC Firms Will Pay for This SaaS

The platform reduces reliance on tribal knowledge, prevents method misuse, speeds up bid-stage hydrology, and produces defensible design notes aligned with Indian practice.

## SaaS Architecture Implication

The true value lies in rule engines, applicability logic, basin-specific defaults, audit-ready explanations, and auto-generated hydrology documentation.