# Hydrology Basin Intelligence Platform – Complete Build Blueprint

## 1. Purpose

This blueprint expands the engineering operating manual into a practical build guide.

It includes:

• Hydrology rule matrix structure

• Example GIS processing code templates

• Example PostGIS spatial schema

• Example project workflow from coordinate to hydrology report

The goal is to enable a small engineering team to begin implementation immediately.

## 2. Hydrology Rule Matrix (Software Encoding Concept)

The hydrology rule matrix converts domain expertise into structured rules that software can evaluate.

Example columns:

• terrain_type

• basin_area_range

• rainfall_zone

• formula_allowed

• formula_rejected

• justification

Example matrix row:

terrain_type: steep_hills

basin_area_range: 0–200 sq km

rainfall_zone: high_monsoon

formula_allowed: Creager, Fuller

formula_rejected: Dicken

justification: steep flashy catchments

Implementation format:

Rules stored in JSON or database tables and evaluated dynamically.

## 3. Example Rule Matrix JSON

{

"rules": [

{

"terrain": "steep_hills",

"basin_area_max": 200,

"allowed_formulas": ["Creager", "Fuller"],

"rejected_formulas": ["Dicken"]

},

{

"terrain": "alluvial_plains",

"basin_area_min": 200,

"allowed_formulas": ["Dicken", "Fuller"],

"rejected_formulas": ["Creager"]

}

]

}

## 4. DEM → Basin Processing Python Template

Example Python workflow using rasterio + whitebox tools

import rasterio

import whitebox

# Load DEM

dem = rasterio.open("dem.tif")

# Fill depressions

wbt = whitebox.WhiteboxTools()

wbt.fill_depressions("dem.tif", "dem_filled.tif")

# Flow direction

wbt.d8_pointer("dem_filled.tif", "flow_direction.tif")

# Flow accumulation

wbt.d8_flow_accumulation("dem_filled.tif", "flow_accumulation.tif")

# Extract streams

wbt.extract_streams("flow_accumulation.tif", "streams.tif", threshold=1000)

# Delineate watershed

wbt.watershed("flow_direction.tif", "outlet_point.shp", "watershed.tif")

## 5. Sub‑Basin Segmentation Concept

Algorithm outline:

1. Identify river network pixels from flow accumulation raster.

2. Detect tributary junction points.

3. Convert river network to vector lines.

4. Split basin polygon at major confluences.

5. Generate sub-basin polygons.

Attributes calculated per sub-basin:

• area

• mean slope

• elevation statistics

• stream order

## 6. PostGIS Spatial Database Example

Example tables with geometry columns

projects

id | latitude | longitude | return_period

basins

id | project_id | geom (POLYGON)

sub_basins

id | basin_id | geom (POLYGON) | area | slope

streams

id | basin_id | geom (LINESTRING)

formula_results

id | sub_basin_id | formula | discharge

reports

id | project_id | file_path

## 7. Example SQL for Spatial Tables

CREATE TABLE basins (

id SERIAL PRIMARY KEY,

project_id INTEGER,

geom GEOMETRY(POLYGON, 4326)

);

CREATE TABLE sub_basins (

id SERIAL PRIMARY KEY,

basin_id INTEGER,

geom GEOMETRY(POLYGON, 4326),

area DOUBLE PRECISION,

slope DOUBLE PRECISION

);

## 8. Example End‑to‑End Workflow

Step 1

Engineer selects location on map.

Step 2

System snaps point to nearest river cell.

Step 3

DEM engine delineates upstream basin.

Step 4

Sub-basins are generated.

Step 5

Terrain metrics computed.

Step 6

Hydrology rule engine selects formulas.

Step 7

Discharge computed per sub-basin.

Step 8

Routing aggregates flows downstream.

Step 9

HFL estimated.

Step 10

Hydrology report generated.

## 9. Example Hydrology Report Contents

• Project location map

• Basin map

• Sub-basin breakdown table

• Applicable formulas

• Discharge estimates

• Final design discharge

• HFL estimate

• Methodology notes

## 10. AI Assisted Engineering Workflow

Recommended workflow

1. Use Claude to design module architecture.

2. Use Cursor IDE to generate scaffolding code.

3. Validate GIS outputs with hydrology expert.

4. Use LLMs for documentation generation.

## 11. First Engineering Milestones

Milestone 1

DEM processing working.

Milestone 2

Watershed delineation API.

Milestone 3

Sub-basin segmentation.

Milestone 4

Hydrology rule engine.

Milestone 5

Report generator.

## 12. Future Enhancements

• Gauge data integration

• HEC‑HMS interoperability

• Satellite rainfall integration

• Machine learning for rainfall estimation