# Hydrology Basin Intelligence Platform – Engineering Operating Manual

## 1. Purpose of this Manual

This document is a practical operating guide for software engineers building the Hydrology Basin Intelligence platform.

It converts the high-level architecture into concrete development tasks, data flows, and implementation steps.

Goal:

Allow a small engineering team (2–4 engineers) to build the first production version within 90 days.

## 2. System Architecture Overview

High-level system components

User Interface (Frontend)

|

v

API Gateway (FastAPI)

|

v

Hydrology Engine

|        |        |

v        v        v

GIS Engine   Rule Engine   Report Generator

|

v

PostGIS Database + Object Storage

Key design principle:

Heavy GIS computation should run as asynchronous workers, not directly inside the API server.

## 3. Data Flow Pipeline

Step 1 – User selects location

User clicks on a map.

Step 2 – API receives coordinates

Coordinates stored in projects table.

Step 3 – GIS worker starts watershed delineation

DEM data processed to compute upstream basin.

Step 4 – Sub-basins created

Watershed divided into hydrologic segments.

Step 5 – Hydrology rule engine selects formulas

Terrain + basin characteristics used.

Step 6 – Discharge estimated

Each sub-basin produces discharge envelope.

Step 7 – Routing model aggregates flows

Downstream peak discharge computed.

Step 8 – HFL estimation

Water level approximated.

Step 9 – Report generated

PDF exported for DPR use.

## 4. DEM Processing Implementation Steps

Step 1: Download DEM data

Sources: SRTM or Copernicus DEM.

Step 2: Merge tiles

Use GDAL merge utilities.

Step 3: Fill sinks

Use WhiteboxTools FillDepressions.

Step 4: Flow direction

Compute using D8 algorithm.

Step 5: Flow accumulation

Generate river network raster.

Step 6: Stream extraction

Apply accumulation threshold.

Step 7: Watershed delineation

Generate upstream basin polygon.

## 5. Basin Segmentation Logic

Goal:

Break large basin into manageable sub-basins.

Algorithm outline

1. Extract river network

2. Detect tributary junctions

3. Assign stream order

4. Split basin at major junctions

5. Create polygon for each sub-basin

6. Compute attributes

Attributes computed

• area

• mean slope

• elevation

• drainage density

## 6. Hydrology Rule Engine Design

Rule engine structure

Input parameters

• basin_area

• terrain_type

• rainfall_zone

• gauge_presence

Rule format example (JSON)

{

"condition": "terrain == 'steep' AND basin_area < 200",

"formula": "Creager"

}

Output

• list of applicable formulas

• rejected formulas

• explanation text

## 7. Flood Discharge Computation

Process

1. Compute discharge for each formula

2. Compute min / max envelope

3. Select conservative design value

Example

Sub-basin 1

Creager Q = 150 m3/s

Fuller Q = 130 m3/s

Design envelope = 130 – 150 m3/s

## 8. Flow Routing Model

Routing objective

Move flows from upstream sub-basins to project site.

Simple routing approach

Q_downstream =

sum(Q_subbasin) - attenuation_factor

Later upgrades

• Muskingum routing

• hydrologic simulation models

## 9. PostGIS Database Schema Example

projects

id | lat | lon | return_period

basins

id | project_id | geometry

sub_basins

id | basin_id | geometry | area | slope

formula_results

id | sub_basin_id | formula | discharge

reports

id | project_id | pdf_path

## 10. APIs to Implement

POST /projects

Create new project

GET /basin/{project_id}

Return basin geometry

GET /subbasins/{project_id}

Return segmented sub-basins

POST /hydrology/run

Trigger hydrology calculations

GET /report/{project_id}

Download generated report

## 11. Developer Workflow (Using AI Tools)

Recommended development workflow

Cursor IDE

Primary coding environment.

Claude

Use for architecture reasoning and code review.

GitHub Copilot

Autocomplete and boilerplate code.

Example workflow

1. Define module specification

2. Ask Claude to outline architecture

3. Implement code in Cursor

4. Validate with hydrology expert

## 12. First 90‑Day Development Plan

Weeks 1–2

Set up repo, database, API skeleton.

Weeks 3–4

DEM processing pipeline.

Weeks 5–6

Watershed delineation.

Weeks 7–8

Sub-basin segmentation.

Weeks 9–10

Hydrology rule engine.

Weeks 11–12

Report generation + pilot testing.

## 13. Example Directory Structure

project-root

backend/

api/

hydrology_engine/

gis_engine/

rule_engine/

report_engine/

frontend/

map_ui/

dashboard/

data/

dem/

rainfall/

scripts/

terrain_processing/

## 14. Glossary

Catchment

Area draining into a river outlet.

DEM

Digital elevation model.

Flow direction

Direction water flows between terrain cells.

Sub-basin

Subdivision of watershed.

Design discharge

Peak flood estimate.

HFL

High flood level.

Routing

Process of combining flows through river network.